Supporting Information for

Smart Gas Sensors: Recent Developments and Future Prospective

Boyang Zong1, 2, Shufang Wu3, Yuehong Yang1, 2, Qiuju Li1, 2, *, Tian Tao1, 2, Shun Mao1, 2, *

1 College of Environmental Science and Engineering, Biomedical Multidisciplinary Innovation Research Institute, Shanghai East Hospital, State Key Laboratory of Pollution Control and Resource Reuse, Tongji University, 1239 Siping Road, Shanghai 200092, P. R. China

2 Shanghai Institute of Pollution Control and Ecological Security, Shanghai 200092, P. R. China

3 Microbiome Medicine Center, Department of Laboratory Medicine, Zhujiang Hospital, Southern Medical University, Guangzhou 510280, P. R. China

*Corresponding authors. E-mail: qjli@tongji.edu.cn (Qiuju Li); shunmao@tongji.edu.cn (Shun Mao)

Supplementary Table

Table S1 Key sensing parameters and definitions in gas sensors [S1-S6]

| Parameter | Formula | Symbol definition |
| --- | --- | --- |
| Sensitivity (Chemiresistor) | Sensitivity=ΔRanalyteR0×100%=Ranalyte-R0R0×100% Sensitivity=ΔIanalyteI0×100%=Ianalyte-I0I0×100% | R0 and I0 are the initial resistance and current of the device under an applied voltage, respectively; and Ranalyte and Ianalyte are the resistance and current upon exposure to analyte, respectively. |
| Sensitivity (Chemical diodes) | Sensitivity=ΔVanalyteV0×100%=Vanalyte-V0V0×100% | Vanalyte and V0 are the applied voltage at a constant current density with and without the analyte, respectively. If the devices are operated at constant voltage, the sensitivity can also be given by ΔI/I. |
| Sensitivity (FETs) | Sensitivity=ΔIDS, analyteIDS=IDS, analyte-IDSIDS×100% | IDS, analyte is the current exposure to analyte and IDS is the initial current at a constant gate-source and drain-source voltage. |
| Sensitivity (Chemical capacitors) | Sensitivity=ΔCC0=Canalyte-C0C0×100% | Canalyte is the capacitance exposure to analyte and C0 is the initial capacitance in farads at a fixed voltage. If the devices are operated at constant capacitance, the sensitivity can also be given by ΔV/V. |
| Sensitivity (Colorimentric sensors) | Sensitivity=ΔCC0=Canalyte-C0C0×100% | Canalyte is a calculated color signal, C0 is its initial state (without analyte). |
| Sensitivity (Colorimetric sensors): Total color difference, or the CIE color parameter (ΔE) | ΔE=L−L02+a−a02+b−b02 | L, a, and b are the initial color values of the sample; and L0, a0 and b0 are the color values of a standard whiteboard (L0=100, a0=0, b0=0). |
| Signal-noise ratio (SNR) | SNRmin=Smin/Navg | Navg is defined as the mean of the standard deviations of 300 data point in the baseline (without analyte); and Smin is defined as the minimum resolvable signal of the sensor. |

Supplementary References

- J. Dai, O. Ogbeide, N. Macadam, Q. Sun, W. Yu et al., Printed gas sensors. Chem. Soc. Rev. 49, 1756-1789 (2020). https://doi.org/10.1039/C9CS00459A

- Z. Meng, R. Stolz, L. Mendecki, K. Mirica, Electrically-transduced chemical sensors based on two-dimensional nanomaterials. Chem. Rev. 119(1), 478-598 (2019). https://doi.org/10.1021/acs.chemrev.8b00311

- R. Paolesse, S. Nardis, D. Monti, M. Stefanelli, C. Di Natale, Porphyrinoids for chemical sensor applications. Chem. Rev. 117(4), 2517-2583 (2017). https://doi.org/10.1021/acs.chemrev.6b00361

- Q. Xu, B. Zong, Q. Li, X. Fang, S. Mao et al., H2S sensing under various humidity conditions with Ag nanoparticle functionalized Ti3C2Tx MXene field-effect transistors. J. Hazard. Mater. 424, 127492 (2022). https://doi.org/10.1016/j.jhazmat.2021.127492

- Z. Li, E. Tian, S. Wang, M. Ye, S. Li et al., Single-atom catalysts: Promotors of highly sensitive and selective sensors. Chem. Soci. Rev. 52, 5088-5134 (2023). https://doi.org/10.1039/D2CS00191H

- T. Minami, Y. Liu, A. Akdeniz, P. Koutnik, N. Esipenko et al., Intramolecular indicator displacement assay for anions: Supramolecular sensor for glyphosate. J. Am. Chem. Soc. 136(32), 11396-11401 (2014). https://doi.org/10.1021/ja504535q